CS 2810 – H.W. #17

The ALU supported set on less than (slt) using just the sign bit of the adder.  Let’s try a set on less than operation using the values -7ten and 6ten.  To make it simpler to follow the example, let’s limit the binary representations to 4 bits:  1001two and 0110two.

1001two – 0110two = 1001two + 1010two = 0011two

This result would suggest that -7 > 6, which is clearly wrong.  Hence, we must factor in overflow in the decision.  Modify the 1-bit ALU in Figure C.5.10 on page C-33 to handle slt correctly.  Make your changes on a photocopy of this figure to save time.